# SPECIFICATION SHEET

#PT361910-1

CASCADE STANDARD AIR SPA - HIGH SEAT

RIGHT SIDE ONLY

STATIONARY AIR SPA:  69 inches long, sanitary gel coated fiberglass tub with three key locked service access doors; seat height 19 ½ inches, tub 30 inches wide, 47 ½ inch deck height, designed so that the user will be in the sitting position at all times. Tub door opening to be 36 inches by 26 ½ inches on hinges capable of holding 250lbs. without deformation. Tub open below to allow access by patient transfer and lift systems. ½ HP regenerative air blower, 11 air jets. Controls to include: pneumatic whirlpool on/off, tub fill and shower; digital tub and fill water temperature indicators; bath oil and shampoo dispensers; safety belt; automatic disinfection and rinse system; thermostatic mixing valve; filler spout; hand-held shower sprayer with vacuum breaker and timer; two inch easy release drain with overflow.

COLOR:     White or Custom

STANDARDS:     UL & CUL Classified

MANUFACTURED:     Aurora, NE. USA

## WARRANTY:    5 year on parts, 6 months mixing valve due to varying water conditions