# SPECIFICATION SHEET

#PT560030-1L

CASCADE COMFORT RECLINING AIR SPA

LEFT SIDE ONLY

STATIONARY AIR SPA:  76 ½  inches long in upright position, 92 inches long in reclined position; low 9 ½ inch leg transfer into spa ; seat height of 21 inches; tub exterior 33 ½  inches wide.  Cabinet 44 ½ inches wide, 50 inches tall. Tub door opening to be 34 inches by 21 inches. All electrical and plumbing are housed in cabinet. Access to spa components available through 3 cabinet doors. Sanitary gel coated fiberglass tub with key locked service access doors Spa designed for easy wheelchair to spa transfer. Spa rated for residents up to 500 pounds. Door mounted with stainless steel hinges and pins capable of holding 250lbs. without deformation. Spa opens below to allow access by patient transfer and lift systems. ½ HP regenerative air blower, 12 air jets.  All spa controls located on cabinet. Controls to include: pneumatic whirlpool on/off, tub fill and shower; digital tub and fill water temperature indicators; disinfection and rinse system; Sporicide injector; thermostatic mixing valve; filler spout; hand-held shower sprayer with vacuum breaker and timer; 1 ½ inch easy release drain with overflow. ¾” Hot and Cold water supply and 1 ½” PVC drain. Available Left Door.

COLOR:     White or Custom Colors

STANDARDS:     UL & CUL Classified

## WARRANTY:    5 year on parts, 6 months mixing valve due to varying water conditions

MANUFACTURED:     Aurora, NE. USA